East Itasca County 1947 ASCS Aerial Photos, Project 163, 1:20,000

Photos on this DVD were taken in early September 1947.  An "ASC 1:20,000" title on a few prints probably indicates they were taken for the Agricultural Stabilization and Conservation Service, U.S. Department of Agriculture; however, the abbreviation may refer to Aero Service Corp., 236 E. Courtland St., Philadelphia PA, whose stamp appears on the reverse of each print.

In general they cover the eastern (4th Principal Meridian) portion of Itasca County, Minnesota.  Another summer 1947 photo project, with project code DET and at a different scale, covers the western portion of the county.  There is some overlap between the two projects.

- Film/filter:  By appearance, Kodak black-and-white Infrared Aerographic film 2424 was used with a Wratten 12 (yellow) filter.  This film/filter combination displays deciduous trees in light tones, and coniferous trees in dark tones.

- Format and scale:  Original negative format was 9" x 9".  Photos were taken at a nominal scale of 1:20,000 (3.17"/mile).  Ground dimensions of each photo are about 2.84 x 2.84 miles.  Photos were taken with about 60 percent overlap along flight lines to provide stereoscopic coverage.

- Flight lines, orientation, labeling:  Flight lines run north-south.  All photo labeling is on the north side of the frame.  This portion of the 1947 Itasca County coverage is unusual in having no roll or line numbering; photos bear only the project number (163) and frame number.

- Scanning:  Photos were scanned in early 2007 from prints filed in Itasca County offices.  Scan intensity was 600 pixels per inch.  Images were saved in JPEG format at an average compression ratio of 10.

The "JPG" directory contains JPEG-format images of all scanned photographs, in numerical order by frame number.  Filenames are in the format ita_163_nnnn.jpg, where:

ita       =   county identifier

163     =    project code

nnnn   =    frame number within project

The "Index Files" directory contains a shapefile showing the center point of every photograph scanned, for use in geographic information systems.  Projection is UTM, Minnesota extended Zone 15.  The "itasca_163_map.jpg" file is an index map made from this shapefile, in a format that can be opened with most image viewing software.