East Itasca 1958 Aerial Photos, 1:20,000

The 581 aerial photos on this DVD were originally taken in early October 1958.  They cover Ranges 22, 23, 24 and a portion of Range 25 (4th Principal Meridian) in Itasca County, Minnesota.  They may be a portion of a larger set.

- Film/filter/season:  Black-and-white panchromatic film was used, probably with a Wratten 12 (yellow) filter to reduce haze effects.

- Format and scale:  Original negative format was 9" x 9".  Photos were taken at a nominal scale of 1:20,000 (about 3.2"/mile.)  Ground dimensions of each photo are about 2.84 x 2.84 miles.  Photos were taken with about 60 percent overlap along flight lines to provide stereoscopic coverage.

- Flight lines, orientation, labeling:  Flight lines run north-south.  All photo labeling is on the north side of the frame.  Photos are numbered by film roll, and by frame number within each roll.  This set has a few gaps in coverage.

- Scanning:  Photos were scanned in March 2007 from prints held by the Itasca County Surveyor's office.  Scan intensity was 600 pixels per inch.  Images were saved in JPEG format at an average compression ratio of 10.  Contrast was slightly enhanced in the scanning process.  Scanned photos are oriented with north upwards.

The "JPG" directory contains JPEG-format images of all scanned photographs, in numerical order by frame number.  Filenames are in the format ita_xx_yyy.jpg, where:

ita    =    project code

xx     =   film roll

yyy   =   frame number within roll

The "Index Files" directory contains a shapefile showing the approximate center point of every photograph scanned, for use in geographic information systems.  Projection is UTM, Minnesota extended Zone 15.  The "east_itasca58_map.jpg" file is an index map made from this shapefile, in a format that can be opened with most image viewing software.