Itasca County Summer-Fall 1959 Aerial Photos, 1:20,000

This set of two DVDs contains a total of 1431 vertical aerial photographs of Itasca County, Minnesota, taken in June-October 1959.   They were taken at 1:20,000 scale on black-and-white panchromatic film; this is typical of photography acquired for the Agricultural Stabilization and Conservation Service, U.S. Department of Agriculture, but it is not known for certain that these photos were taken for that agency.   The project covers the entire county; a few frames are missing from this set.

Another photo project was taken in the western portion of the county in summer of the same year for the USDA Forest Service; it was flown at 1:15,840 scale with black-and-white infrared film and bears the project code DET.

- Film/filter:  This project was probably flown with Kodak Double-X Aerographic Film 2405, and a Wratten 12 (yellow) filter for haze reduction.

- Format and scale:  Original negative format was 9" x 9".  Photos were taken at a nominal scale of 1:20,000 (3.17"/mile), from an altitude of about 10,000 feet above terrain, with a 6" focal length camera (CFL = 153.08mm.)  Ground dimensions of each photo are about 2.84 x 2.84 miles.  Photos were taken with about 60 percent overlap along flight lines to provide stereoscopic coverage.

- Flight lines, orientation, labeling:  Flight lines run east-west.  These DVDs display all photos in north-up orientation.  All labeling is on the west side of the photo frame.  Labeling on photos includes date of flight, project code, film roll number, and frame number.

- Scanning:  Photos were scanned in 2007 from prints filed in Itasca County offices.  Scan intensity was 600 pixels per inch.  Images were saved in JPEG format at an average compression ratio of 10.

The "JPG" directory contains JPEG-format images of all scanned photographs, in numerical order by roll and frame number. Disk 1 contains 641 frames from rolls 1-4, and Disk 2 contains 790 frames from rolls 5-9.  Filenames are in the format ita_xx_yyy.jpg, where:

ita       =   county identifier and project code

xx       =    film roll number

yyy     =    frame number within film roll

The "Index Files" directory contains a shapefile showing the approximate center of every photograph scanned, for use in geographic information systems.  Projection is UTM, Minnesota extended Zone 15.  The "itasca59_20K_map.jpg" file is an index map made from this shapefile, in a format that can be opened with most image viewing software.