Wright County 1957 ASCS/CSS Aerial Photos, 1:20,000

The 465 aerial photos on this DVD were originally taken in 1957 for the Commodity Stabilization Service, predecessor of the Agricultural Stabilization and Conservation Service, U.S. Department of Agriculture.

- Film/filter/season:  Black-and-white panchromatic film was used, probably with a Wratten 12 (yellow) filter to reduce haze effects.  Photo season was unusually long, from May 6 through October 28, 1957.

- Format and scale:  Original negative format was 9" x 9".  Photos were taken at a nominal scale of 1:20,000 (about 3.2"/mile.)  Ground dimensions of each photo are about 2.84 x 2.84 miles.  Photos were taken with about 60 percent overlap along flight lines to provide stereoscopic coverage.

- Flight lines, orientation, labeling:  Flight lines run east-west.  All photo labeling is on the west side of the frame.  Photos are numbered by film roll, and by frame number within each roll.  This set has a few minor coverage gaps.

- Scanning:  Photos were scanned in March 2007 from prints filed in Wright County Soil and Water Conservation District offices.  Scan intensity was 600 pixels per inch.  Images were saved in JPEG format at an average compression ratio of 10.  Contrast was slightly enhanced in the scanning process.  Scanned photos are oriented with north upwards.

The "JPG" directory contains JPEG-format images of all scanned photographs, in numerical order by frame number.  Filenames are in the format wx_ym_zzz.jpg, where x, y and z are variables:

wx    =     project code  (wn and wp both used in this project)

yt     =   film roll

zzz   =     frame number within roll

The "Index Files" directory contains a shapefile showing the center point of every photograph scanned, for use in geographic information systems.  Projection is UTM, Minnesota extended Zone 15.  The "wright_ascs57_map.jpg" file is an index map made from this shapefile, in a format that can be opened with most image viewing software.