Wright County 1963 ASCS Aerial Photos, 1:20,000

Photos on this DVD were taken on June 12, 13, 20 and 23, 1963 for the Agricultural Stabilization and Conservation Service, U.S. Department of Agriculture.

- Film/filter:  Three rolls of black-and-white panchromatic film were used, probably with a Wratten 12 (yellow) filter to reduce haze effects.

- Format and scale:  Original negative format was 9" x 9".  Photos were taken at a nominal scale of 1:20,000 (about 3.2"/mile), using a lens with a calibrated focal length of 209.33mm (nominally 8 ¼") from approximately 14,750' MSL.  Ground dimensions of each photo are about 2.84 x 2.84 miles.  Photos were taken with about 60 percent overlap along flight lines to provide stereoscopic coverage.

- Flight lines, orientation, labeling:  Flight lines run east-west.  All photo labeling is on the west side of the frame.  Photos are numbered by film roll, and by frame number within each roll.

- Scanning:  Photos were scanned in early 2007 from prints filed in Wright County Soil and Water Conservation District offices.  Scan intensity was 600 pixels per inch.  Images were saved in JPEG format at an average compression ratio of 10.  Contrast was slightly enhanced in the scanning process.

The "JPG" directory contains JPEG-format images of all scanned photographs, in numerical order by frame number.  Filenames are in the format wp_xdd_yyy.jpg, where:

wp    =     project code identifier

xdd     =   roll number

yyy   =     frame number within roll

The "Index Files" directory contains a shapefile showing the center point of every photograph scanned, for use in geographic information systems.  Projection is UTM, Minnesota extended Zone 15.  The "wright_ascs63_map.jpg" file is an index map made from this shapefile, in a format that can be opened with most image viewing software.