Wright County 1970 ASCS Aerial Photos, 1:40,000

The aerial photos on this DVD were originally taken in early September 1970 for the Agricultural Stabilization and Conservation Service, U.S. Department of Agriculture.  Each of the 237 images is one-quarter of an original frame of photography.

- Format and scale:  Original 9" x 9" negatives were taken at a nominal scale of 1:40,000 (about 1.6"/mile), probably with a camera having a 152mm (6") nominal focal length, from an altitude of 20,000 feet above ground.  This set consists of quarter-frame enlargements made from alternate frames along each flight line; consequently it does not provide stereoscopic coverage.

- Film/filter/season:  Black-and-white panchromatic film was used, probably with a Wratten 12 (yellow) filter to reduce haze effects.

- Flight lines, orientation, labeling:  Flight lines run east-west.  All photo labeling is on the west side of the image.  Images are labeled by film roll, frame number within roll, and quarter-frame within the original frame.  This set has a few minor coverage gaps.

- Scanning:  Photos were scanned in March 2007 from 1:15,000 scale 12" x 12" quarter-frame enlargements filed in Wright County Soil and Water Conservation District offices.  Scan intensity was 600 pixels per inch.  Images were saved in JPEG format at an average compression ratio of 10.  Photos are oriented with north upwards.

The "JPG" directory contains JPEG-format images of all scanned photographs, in numerical order by frame number.  Filenames are in the format wp_xll_yyyz.jpg, where x, y and z are variables:

wp    =     project code

xll     =    film roll

yyy   =     frame number (alternate frames only)

z       =     quarter-frame (a = NW, b = NE, c = SE, d = SW)

The "Index Files" directory contains a shapefile showing the center point of every quarter-frame scanned, for use in geographic information systems.  Projection is UTM, Minnesota extended Zone 15.  The "wright_ascs70_map.jpg" file is an index map made from this shapefile, in a format that can be opened with most image viewing software.